| CHECK LIST FOR BITUMINOUS WORKS (DBM/BC) Format No: CTP/QC/CL/001 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Chainage: Description of Work |  |  | Date : RFI No : |  |  |  |  |
| S.No. | Description |  | Yes | No |  | NA |  |
| 1 | Confirms to job mix formula |  |  |  |  |  |  |
| 2 | Free from deleterious materials |  |  |  |  |  |  |
| 3 | Spread layer thickness is acceptable |  |  |  |  |  |  |
| 4 | Temperature at break down rolling is within specs |  |  |  |  |  |  |
| 5 | No segregation during transport and paving |  |  |  |  |  |  |
| 6 | Previous layer approved / Surface regularity test is found OK |  |  |  |  |  |  |
| 7 | Weather is foggy, rainy, dusty or windy |  |  |  |  |  |  |
| 8 | Tack coat required (if yes follow the check list for Prime/Tack coat) |  |  |  |  |  |  |
| 9 | Adequate Machinery available |  |  |  |  |  |  |
| 10 | Mix transported to site are covered properly |  |  |  |  |  |  |
| 11 | Thermometer for recording temperature is available |  |  |  |  |  |  |
| 12 | Temperature record sheet available / Trip record sheet available |  |  |  |  |  |  |
| 13 | Survey stakes set out to determine vertical and horizontal control of asphalt layers |  |  |  |  |  |  |
| 14 | Lines and Levels are within tolerance |  |  |  |  |  |  |
| 15 | Temperature maintained as per MORTH spec. |  |  |  |  |  |  |
| 16 | Are the finish levels of asphalt in conformance with tolerable specification limits |  |  |  |  |  |  |
| 17 | Samples tested at lab are found OK |  |  |  |  |  |  |
| Remarks: |  |  |  |  |  |  |  |

| CHECK LIST FOR EARTHWORK Format No: CTP/QC/CL/002 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RFI No: Chainage / Layer : |  |  | Description of Work: Date : |  |  |  |  |
| S.No | Description |  | Yes |  | No | NA |  |
| 1 | Check material is from approved borrow |  |  |  |  |  |  |
| 2 | Free from roots and other perishable material |  |  |  |  |  |  |
| 3 | Grading is acceptable |  |  |  |  |  |  |
| 4 | Spread layer thickness is acceptable |  |  |  |  |  |  |
| 5 | Moisture is near optimum prior to compaction |  |  |  |  |  |  |
| 6 | Lines/Levels are within tolerance |  |  |  |  |  |  |
| Remarks: |  |  |  |  |  |  |  |

| CHECK LIST FOR GSB/WMM/CTSB |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| RFI No Chainage / Layer |  |  | Description of Work Date : |  |  |  |  |
| S.No | Description |  | Yes | No |  |  | NA |
| 1 | Confirms to job mix formula |  |  |  |  |  |  |
| 2 | Free from deleterious materials |  |  |  |  |  |  |
| 3 | Spread layer thickness is acceptable |  |  |  |  |  |  |
| 4 | Moisture is near optimum prior to compaction |  |  |  |  |  |  |
| 5 | No segregation during transport and paving |  |  |  |  |  |  |
| 6 | Lines and Levels are within tolerance |  |  |  |  |  |  |
| Remarks: |  |  |  |  |  |  |  |

| CHECK LIST FOR THERMOPLASTIC PAINT |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Location : |  |  |  | Date: |  |  |  |  |
| Layer: |  |  |  | RFI No.: |  |  |  |  |
| S. No. | Description |  |  | Yes |  | No |  | NA |
| 1 | Whether the road surface approved for road marking |  |  |  |  |  |  |  |
| 2 | Whether the road surface has been thoroughly cleaned of dust/dirt/grease/oily matter or other foreign matter |  |  |  |  |  |  |  |
| 3 | Whether the pavement temperature checked and found to be more than 10 o C |  |  |  |  |  |  |  |
| 4 | Whether arrangements for diversion of traffic made |  |  |  |  |  |  |  |
| 5 | Whether temperature of the paint at the time of application checked as per specification of the manufacture |  |  |  |  |  |  |  |
| 6 | Is the time duration during which the paint remains in molten state checked to be not more than 4 hours |  |  |  |  |  |  |  |
| 7 | Whether the paint has been maintained within the manufacturers specified range during the application |  |  |  |  |  |  |  |
| 8 | Whether thickness of applied paint checked as per BS-3262(part-3) and found to be minimum 2.5 mm |  |  |  |  |  |  |  |
| 9 | Whether additional glass beads of grading type 2 as per MOSRTH Clause 803, have been sprayed in case of manual application for arrows & letters etc @ 250 g/sq.m of area |  |  |  |  |  |  |  |
| 10 | Whether the surface is level, uniform and free from surface |  |  |  |  |  |  |  |
| Remarks: |  |  |  |  |  |  |  |  |

| CHECK LIST FOR CONCRETING |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location : |  |  | Date | : |  |
| Layer: |  |  | RFI No. | : |  |
| S. No. | Description |  | Yes | No | NA |
| 1 | Concerned drawing available |  |  |  |  |
| 2 | Strata/levels found acceptable |  |  |  |  |
| 3 | Mix and its materials approved |  |  |  |  |
| 4 | Formwork/scaffolding approved |  |  |  |  |
| 5 | Bar bending schedule / Reinforcement approved |  |  |  |  |
| 6 | Adequate cover blocks/chairs/spacers provided |  |  |  |  |
| 7 | Slump found within the range |  |  |  |  |
| 8 | Required cubes casted |  |  |  |  |
| 9 | Placing/compaction/finishing/workmanship for concrete found acceptable |  |  |  |  |
| 10 | Pre-stressing sheathing approved |  |  |  |  |
| 11 | Required equipment/lighting/communication provided |  |  |  |  |
| 12 | Concrete conveying arrangement found acceptable |  |  |  |  |
| 13 | Curing arrangement made |  |  |  |  |
| Remarks: |  |  |  |  |  |

| CHECK LIST FOR REINFORCEMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location : |  |  | Date: |  |  |
| Layer: |  |  | RFI No.: |  |  |
| S.No. | Description |  | Yes | No | NA |
| 1 | Concerned drawing available |  |  |  |  |
| 2 | Materials approved |  |  |  |  |
| 3 | Bar Bending Schedule approved |  |  |  |  |
| 4 | Bar length, diameter as per working drawing and schedule |  |  |  |  |
| 5 | Lap length found satisfactory |  |  |  |  |
| 6 | Corrosion treatment of bars, if required, satisfactory |  |  |  |  |
| 7 | Adequate cover blocks/chairs/spacers provided |  |  |  |  |
| 8 | Bar tied with binding wire rigid and adequate |  |  |  |  |
| Remarks: |  |  |  |  |  |